Optimization of CPU Core Processing for

High Data Rate QPSK Communication

1. Introduction

- High data rate communication requires significant computational resources.

- GNU Radio flowgraphs contain multiple DSP blocks that process samples in real time.

- Efficient CPU utilization is necessary to avoid processing bottlenecks.

2. Existing Challenge

- Limited CPU core utilization.

- Some blocks became processing bottlenecks.

- Higher sample rates increased computational load.

- Possibility of buffer overflow and dropped samples.

- System initially used a USB 2.0 connection.

Problem Faced

During high data-rate operation, the GNU Radio flowgraph experienced:

- Underflow issues

- Bandwidth limitations

- Sample rate mismatches

- Reduced system stability

3. CPU Intensive Blocks

The following blocks were identified as the major contributors to processing load:

- USRP Source

- Symbol Synchronization

- Costas Loop

- QPSK Demodulator

- Frame Extractor

- Phase Resolver

4. USB 3.0 Configuration

- The SDR was connected through a USB 3.0 port.

Benefits

- Higher data transfer rate

- Continuous sample streaming

- Reduced packet loss

- Improved real-time performance

USB 2.0 vs USB 3.0 — Comparison

| Feature | USB 2.0 | USB 3.0 |
| --- | --- | --- |
| Release Year | 2000 | 2008 |
| Maximum Data Transfer Rate | 480 Mbps (0.48 Gbps) | 5 Gbps |
| Real-World Speed (Typical) | 30–40 MB/s | 350–450 MB/s |
| Bus Bandwidth | 480 Mbps | 5 Gbps |
| Duplex Mode | Half-Duplex | Full-Duplex |
| Number of Data Lanes | 1 | 2 (SuperSpeed lanes) |
| Power Supply | 5V, up to 500 mA | 5V, up to 900 mA |
| Connector Color | Black | Blue |
| Cable Wires | 4 wires | 9 wires |
| Backward Compatibility | N/A | Yes (works with USB 2.0 devices) |
| Forward Compatibility | No | N/A |
| Use Cases | Keyboards, mice, printers, basic storage, low-speed peripherals | External HDD/SSD, high-speed storage, video capture, large file transfer, SDR devices |
| Example in SDR (USRP) | Suitable for low sample rates (≤ 5–10 MSPS) | Required for higher sample rates (≥ 10–50 MSPS and above) |

In brief: this section covers the fix for the I/O side — moving to USB 3.0 removed the transfer-rate ceiling that was causing packet loss.

5. Bandwidth and Sample Rate Optimization

Parameter Tuning

- SDR parameters adjusted to ensure proper operation: Bandwidth = Sample Rate.

- Example: Sample Rate = 15 MSps, Bandwidth = 15 MHz.

Benefits

- Efficient spectrum capture

- Reduced processing overhead

- Improved signal quality

6. Underflow Analysis

Observation

- The GNU Radio flowgraph operates correctly at 5 MSps.

- No underflow events are observed at 5 MSps.

- When the sample rate is increased to 15 MSps, underflow events occur.

Performance Comparison

| Sample Rate | Status |
| --- | --- |
| 5 MSps | Working Correctly |
| 15 MSps | Underflow Occurs |

Reason

- At 15 MSps, three times more samples must be processed every second.

- The processing chain cannot sustain the required throughput continuously.

- CPU, USB interface, or DSP blocks become bottlenecks, causing underflow.

7. Overflow Problem

- Overflow occurred when the receiver could not process incoming samples fast enough.

- Data accumulated in buffers, causing sample loss.

- This affected real-time communication performance.

Root Causes

- High incoming data rate

- CPU processing bottleneck

- Inefficient resource utilization

Solution Implemented

- Optimized GNU Radio flowgraph

8. Hardware and Simulation Validation (htop CPU Activity)

- System validation was carried out both on real USRP hardware and through simulation.

- htop CPU monitoring confirmed multiple active cores during execution in both setups.

Purpose of This Check

This section exists to confirm, with real operating-system evidence, that the flowgraph was actually executing in parallel — not just configured to. The htop screenshots above show several cores simultaneously active at non-trivial load, rather than a single core pegged at 100% while the rest sit idle, which would indicate GNU Radio had silently fallen back to single-core scheduling.

Cores Used by Our Flowgraph

The flowgraph distributes its main processing stages across four dedicated CPU cores:

| Core | Task in Our Flowgraph |
| --- | --- |
| Core 1 | USRP Source — receives raw RF/I-Q samples from the SDR |
| Core 2 | Multiply Const — amplifies/scales the incoming samples |
| Core 3 | Keep M in N — extracts the fixed-size packet/chunk from the stream |
| Core 4 | Stream Mux — recombines the processed chunks into one output stream |

This is a pipeline, not just parallel chunks: each core works on a different stage of the same sample at the same time — Core 1 receives new samples while Core 3 extracts older packets and Core 4 combines already-finished ones. That overlap is what lets the system keep up with a continuous 15 MSps stream instead of stalling between stages.

The htop screenshots above are the proof of this concurrent pipeline. The Section 9 per-core table extends this picture to all 16 logical cores and shows the real imbalance: cores 4–6 (the Symbol Sync / Costas Loop chains) carried the heaviest load, while most other cores were nearly idle — meaning there is still headroom to assign more DSP blocks to unused cores, exactly the lesson Section 14's reference design (Miller's 8-core affinity table) demonstrates at a larger scale.

9. Per-Core CPU Utilization

Observed utilization across all 16 logical CPU cores during high data-rate processing:

| CPU Core | Utilization (%) | Status |
| --- | --- | --- |
| Core 0 | 3.8 | Low |
| Core 1 | 8.7 | Low |
| Core 2 | 29.6 | Moderate |
| Core 3 | 19.5 | Moderate |
| Core 4 | 70.8 | High |
| Core 5 | 52.0 | High |
| Core 6 | 51.9 | High |
| Core 7 | 17.6 | Moderate |
| Core 8 | ~2 | Low |
| Core 9 | ~5 | Low |
| Core 10 | ~7 | Low |
| Core 11 | 5.8 | Low |
| Core 12 | 1.3 | Low |
| Core 13 | 8.4 | Low |
| Core 14 | 1.3 | Low |
| Core 15 | 5.2 | Low |

- A small subset of cores (4, 5, and 6) carried the highest load, confirming that the heaviest DSP blocks were concentrated on a few cores.

- Remaining cores operated at low-to-moderate utilization, indicating room for further load balancing.

10. Hardware Setup — USRP Connection

- The NI USRP-2901 was connected via SMA cables for transmit and receive paths, interfaced over USB 3.0.

11. GNU Radio Flowgraph — Simulation Configuration

- The simulation flowgraph used a UHD: USRP Source/Sink, Multiply Const, Keep M in N, and Stream Mux blocks to replicate the chunk-based parallel processing chain.

- Sample rate was configured at 15 MSps with a center frequency of 80 MHz.

12. Simulation Output — Constellation Diagram

- The simulation (“SIMULATION_NO_LIMESDR_HighDataRate_UNcoded_QPSK”) was executed to verify correct signal flow before hardware deployment.

- The resulting I-Q constellation plot confirmed the expected QPSK phase clustering.

13. Hardware Execution — USRP Flowgraph

- The validated flowgraph (“HighDataRate_UNcoded_QPSK with accurate samples”) was executed on hardware using the USRP source and sink at 15 MSps.

- Terminal logs confirmed successful generation and execution of the compiled QPSK 15 Mbps processing chain.

CPU Core

| Flowgraph Block | Assigned Core |
| --- | --- |
| Test Modulator File Source Block | 1 |
| LimeSuite Source (Receiver) | 1 |
| LimeSuite Sink (Modulator) | 1 |
| Front-End Skip and Multiplier Blocks | 2 |
| Front-End “Keep M in N” and “Stream Mux” Blocks | 2 |
| Front-End File Source (Preamble) Block | 2 |
| Symbol Synchronizer / Costas Loop — Chunk Chain #1 | 4 |
| Symbol Synchronizer / Costas Loop — Chunk Chain #2 | 5 |
| Symbol Synchronizer / Costas Loop — Chunk Chain #3 | 6 |
| “Complex To Float” Blocks | 4, 5, 6 |
| “Binary Slicer” and “Interleave” Blocks | 7 |
| “TAG_CHUNKpreamble” Blocks | 7 |
| “Chunk_ExtractQPSK” Blocks | 7 |
| Back-End “Stream Mux” Block | 3 |
| Back-End “Keep M in N” Block | 3 |
| “Tag_FrameASM” Block | 3 |
| “Extract_Frame” Block | 3 |
| “Resolve_Phase” Block | 3 |
| Back-End File Sink Block | 3 |

Key Test Parameters

| Parameter | Value |
| --- | --- |
| RF Center Frequency | 80.0 MHz |
| Sample Rate | 15.0 MSps |
| Symbol Sync Input | 2.0 samples/symbol |
| Symbol Sync Output | 1.0 sample/symbol |
| Costas Loop Order | 4 (QPSK) |

Results:

- The demodulator recovered the transmitted frame stream with no bit errors, no missing frames, and no extra frames or bits.

- The 15.0 Mbps frame stream was successfully reassembled in real time from the three parallel chunk chains.

- Symbol Synchronizer and Costas Loop lock were continually re-established for every new chunk and chunk-preamble marker.

- Setting a high minimum output-multiple value for the custom blocks, so that each scheduler work-call processed 15–20 frames at a time, improved flowgraph throughput at high data rates.